COURSE REPORT: DYNAMICS OF ELECTRICAL MACHINES AND DRIVES

CONTROL OF A SE DC FOR ATM TRAMWAY VEHICLE “CARRELLI 1928”

Student: Lorenzo Berti (10533775)

PROBLEM DATA AND PLANT ANALYSIS

We can divide the problem into three parts: excitation circuit, armature circuit and mechanical load.

- Mechanical load:

(A) dynamic equilibrium of the train

Calculating the momentum referred to the point P (the contact point between the disk and the plane) we obtain, looking at the image (A):

Mp:		 Tⅇ-MgsinθR-βω=Jω + MRx		(dynamic equilibrium)

x=Rω					(for non-slipping condition)

Tⅇ-MgsinθR-βω=(J+MR2)ω	(1)

MgsinθR=Tl 	(which is the load due to the sloping plane)

Where 		M=Mt+NpMp	(Mt is the mass of the train, Np number of people,

Mp average mass of each person)

R=vnωn

From (1) eq I can compute the transfer function of the mechanical load:

Te-Tl1Jeqs+β=w 		Gw=1Jeqs+β 	(2) 	(transfer function of the mechanical load)

I need to know the two parameters Jeq and β:

Jeq: 		   Jp=J+MR2

Where		 J=12mR2	(the moment of inertia of a solid homogeneous disk with mass m)

Considering M≫m I can neglect J in the computation of the inertia

So 	 Jeq=MR2

β:		β=Tfωn

%mechanical load

wn= 314;%rad/s

vn= 60*(1000/3600); %m/s

R=vn/wn; %m

M=10*1000+200*80; %kg

an=60*(1000/3600)/25;%m/s^2

Ft=M*an;%N

Ff=(1/3)*Ft; %N

Tf=Ff*R;%Nm

beta=Tf/wn;%Nm/s

J=M*R^2;%kg*m^2

Where 		Tf=FfR=13FtR=13ManR

(B) it’s the portion of the MATLAB code in which the data regarding the mechanical load are computed

- Excitation circuit:

(C) excitation circuit

Analysing the excitation circuit in picture (C) I can calculate the transfer function for the excitation current:

Vⅇn=Reie+Lepie	(Kirchhoff voltage’s law)

Ven1sLe+Re=ie		(in frequency domain)

Gie=1sLe+Re		(transfer function) (3)

Where 		Re=VenIen		And 		Le=τeRe

%excitation circuit

tau_e=1; %s

Ien= 1; %A

Ven= 120; %V

Re= Ven/Ien;%Ohm

Le= tau_e*Re;%H

(D) it’s the portion of the MATLAB code in which the data regarding the excited circuit are computed

- Armature circuit

(E) power balance of the armature circuit

The transfer function can be calculated by analizing the armature curcuit:

Va=Raia+Lapia 		(Kirchhoff voltage’s Law)

Va-E1Ra+Las=ia		(in frequency domain)

Gia= 1Ra+Las			(tranfer function) (4)

To calculate Ra,La I need to analize the power balance

η=PnPel

Pn=Pt+Pf=Ft+Ffvn 	(the power required by the mechanical load is determined by the traction and friction forces)

1-ηPel=PJ=Raian2 	(the power which is not converted into mechanical power 1-ηPel is the power lost due to Joule losses PJ=Raian2)

So	Ra= 1-ηPelian2 	(where ian=PelVan )

And 	La=τaRa

%armature circuit

Pn=(Ft+Ff)*vn;%W

Pel=Pn/0.9; %W

Van=600; %V

Ian=Pel/Van; %A

En=Pn/Ian;%V

K=En/(Ien*wn);%ohm/s

Tn=Pn/wn;%Nm

Ks=Tn/(Ian*Ien);

Ra=0.084; %0.1*Pn/(Ian^2);%ohm

tau_a=0.010; %s

La=tau_a*Ra;%H

(F) it’s the portion of the MATLAB code in which the data regarding the armature circuit are computed

CONTROL DESIGN

The three plants that I need to control are (2), (3), (4):

Gw=1JPs+β 	(2)			Gie=1sLe+Re	(3)			Gia= 1Ra+Las	(4)

I used the traditional speed control scheme for separatly excited DC machine:

(G) Speed control scheme of a separately excited DC machine

(H) simulink implementation of the problem and control

I chose PID control strategy and I used the “pidtune” function in matlab in order to obtain Kp, Ki through the specifications Pm and wc which I selected trying and failing on simulink simulation

%w requirements

wc_w= 7;

Pm_w= 90;

%Ia requirements

wc_Ia= wc_w*10;

Pm_Ia= 80;

%Ie requirements

wc_Ie= 0.9;

Pm_Ie= 90;

%% REGOLATORs

%w regulator

opt_w=pidtuneOptions('PhaseMargin', Pm_w);

[Reg_wpi, info_w]=pidtune(Gw,'PI',wc_w,opt_w);

Reg_w=tf(Reg_wpi);

%Ia regulator

opt_Ia=pidtuneOptions('PhaseMargin', Pm_Ia);

[Reg_Iapi, info_Ia]=pidtune(GIa,'PI',wc_Ia,opt_Ia);

Reg_Ia=tf(Reg_Iapi);

%Ie regulator

opt_Ie=pidtuneOptions('PhaseMargin', Pm_Ie);

[Reg_Iepi, info_Ie]=pidtune(GIe,'PI',wc_Ie,opt_Ie);

Reg_Ie=tf(Reg_Iepi);

I implemented the controllers in simulink using the “PID controller” block and their relative antiwindup strategy due to the saturation of Va,Ve which can’t be greater than Van,Ven because of Power supply limitations.

In addiction, please notice that I selected the armature current controller bandwidth 10 times the one of the angular speed because it is necessary to design separately the two controller (cascade control loop).

RESULTS AND DISCUSSION

Operating region

- Operating region

I computed the operating region to be sure that in steady state conditions nothing goes over the rated values.

It’s interesting to notice how in flux weakening region the torque and the armature current decrease, while the armature voltage keeps constant.

Notice that the exciting current is constant in costant torque region, and it decreases in flux weakening region (I multiplied the Ie values by 400 just to visualize the results in the graph).

Angular speed control

(J) angular speed control

The response is a little bit slow and with some overshoot, it comes out from some compromise between a sufficiently quick response and a not too aggressive controller to avoid high changes in the torque. However, the controller is stabilizing, and it is allowed by the tram to go quicker for a short period of time and for small values.

Torque and armature current control

(K) torque control

(L) armature current control

The two graphs are quite similar due to the proportional relation: T=K(ie)*Ia.

We can see the effect of the antiwindup scheme which limits the output when the reference saturates. there are no huge quick changes in the torque which can generate vibration and wear.

Armature voltage control

(M) armature voltage control

The armature voltage has the stricter requirement concerning saturation point due to the power supply limitations. while for armature current and torque it is possible to go above the rated value for a short time, regarding the armature voltage this is not possible. We can notice the effect of the antiwindup strategy at time 300s.